# Lexus LS 430 Specifications & Features*

# Dimensions

| Body Type | 4-door 5-passenger luxury sedan. |
| --- | --- |
| Construction | Unitized body with anti-vibration subframes. Extensive use of galvannealed steel and other anti-corrosive measures. |
| Overall Length | 4995mm (196.7 in.) |
| Width | 1830mm (72.0 in.) |
| Height (unloaded) | 1490mm (58.7 in.) |
| Height (unloaded with air suspension) | 1470mm (57.9 in.) |
| Wheelbase | 2925mm (115.2 in.) |
| Curb Weight | 1795 kg (3957 lb) |
| Tread (front/rear) | 1570/1570 mm (61.8/61.8 in.) |
| Tread (front/rear with air suspension) | 1575/1575 mm (62.0/62.0 in.) |
| Headroom (front/rear) | 967/964 mm (38.1/38.0 in.) |
| Legroom (front/rear) | 1118/954 mm (44.0/37.6 in.) |
| Shoulder Room (front/rear) | 1481/1479 mm (58.3/58.2 in.) |
| Trunk Space | 0.571 cu. m (20.2 cu. ft.) |
| Fuel Tank Capacity | 84.0 L (18.5 imp. gal.) |

# Engine

| Engine Type | (3UZ-FE) V8 |
| --- | --- |
| Displacement | 4.3 litres (4293 cc) |
| Valvetrain | DOHC 32 valves, Variable Valve Timing with intelligence (VVT-i) |
| Acoustic Control Induction System (ACIS) and Electronic Throttle Control System with intelligence (ETCS-i), Stainless Steel Exhaust System |  |
| Bore x Stroke | 91.0 x 82.5 mm (3.58 x 3.25 in.) |
| Horsepower | 290 hp (216 kW) @ 5600 rpm |
| Torque | 320 lb. ft. (434 N.m) @ 3400 rpm |
| Compression Ratio | 10.5:1 |
| Engine Management | Computer control of sequential multi-point fuel injection. |
| Direct Ignition System (DIS) with dual knock sensors. |  |
| Fuel Requirement | Premium Unleaded recommended. |
| Ultra Low Emissions Vehicle (ULEV) |  |

# Drivetrain

| Transmission | 5-speed Super Electronically Controlled Transmission (Super ECT), Transmission Cooler, Overdrive top gear. |
| --- | --- |
| Drive Wheels | Rear-wheel drive with Traction Control system (TRAC) |
| Final Drive Ratio | 3.266:1 |
| Suspension | 4-wheel independent, double wishbone suspension, coil springs, low pressure gas shock absorbers, and front and rear stabilizer bars. Anti-dive and anti-squat geometry, anti vibration subframes. |
| Steering | Vehicle-speed-sensing electronically-controlled Progressive Power Steering (PPS), power rack-and-pinion steering, 3.53 turns, lock-to-lock. |
| Brakes | 4-wheel, power assisted, front and rear ventilated disc brakes. |
| Antilock Braking System (ABS) | 4-sensor, 4-channel. |
| Brake Assist, Electronic Brake force Distribution (EBD) |  |

# Performance Capabilities

| Acceleration, 0-60 mph** | 6.3 seconds |
| --- | --- |
| Fuel Consumption, L/100 km*** | 12.8 City, 8.8 Highway, 11.0 Combined |
| Turning Circle | 10.7 m (35.1 ft.) |
| Coefficient of Drag (cd.) | 0.25 with air suspension 0.26 without air suspension |

# Exterior Features

Low reflection windshield with heating system and 3-jet washer nozzles

Integrated headlight washer

Helical-coil type short-pole antenna (mounted on the rear centre of the roof)

Wheels/Tires – 7-spoke 17x 7.5 JJ alloy wheels. P225/55R17 performance all-season radials (full-size spare tire).

Stainless steel exhaust system

Clearance and Back-up sonar

# Interior Features

Leather package: Seats and headrests, armrest. Leather and wood steering wheel, shift knob and door trim.

Power-assisted driver and front passenger heated seats (Adjustments: 14-way driver, 10-way passenger). Headrest –vertical and tilt.

Lexus Driver Memory system includes three driver memory settings for power adjustments on driver’s seat, side mirrors, power tilt-and-telescopic, 4-spoke steering wheel and adjustable shoulder belt anchor

Maple or Walnut wood trim accents

Audio system: Lexus Premium AM/FM with cassette, 178-watt sound system with 7 speakers and Radio Data System (RDS)

In-dash 6-disc CD auto-changer, diversity, roof mounted antenna.

Electronic analog gauges with multi-function information

Display (Outside temperature gauge and Vehicle performance information)

Automatic air conditioning with dual zone climate control and manual override, independent driver and front passenger temperature soft-touch controls with dust, pollen and deodorizing air filter.

High solar energy absorbing tinted glass

Air quality control system: Exhaust gas and NOx sensors

Front centre console automatic swing register

Power trunk lid with auto close system

Rear window power sunshade

Power fuel filler door release

Power windows with driver and passenger “auto up/down” feature with jam protection

Power tilt-and-telescopic steering wheel

Power tilt-and-slide moonroof with sunshade

Dual heated power remote-controlled and colour-keyed side mirrors with electrochromic automatic dimming

Automatic dimming electrochromic day/night inside rearview mirror

Heavy duty rear window defogger with timer

Digital clock

Retained accessory power

HomeLink universal transceiver, integrated garage door opener

Upper and lower glove boxes

# Convenience Features

Cruise control

Park assist

Adjustable rear seat headrests

Power driver and front passenger adjustable pillar shoulderbelt anchors

Front seatback pockets

Rear seat fold-down armrest/storage console with trunk pass-through access panel

Electronic compass in rearview mirror

Accessory power outlets

Driver and passenger illuminated vanity mirrors with ticket holder

Front and rear cupholders

Low washer fluid, engine oil level, fuel and coolant level, door ajar and pressure warning lamps

Power door locks with driver’s door 2-turn unlock feature

Map lamps

Reversible trunk mat

Two-level console box with sliding upper tray

Storage drawers under front seats, expandable front door pockets

Illuminated entry system

Carpeted floor mats

Cargo net

Automatic Light control system

Wheel-locks

# Safety Features

Antilock Braking System (ABS) – 4-sensor, 4-channel.

Brake Assist

Electronic Brake force Distribution (EBD)

Traction Control (TRAC)

Vehicle Skid Control (VSC)

High Intensity Discharge (HID) headlamps with auto levelling system

Fully-integrated anti-theft deterrent system with encoded vehicle immobilizer

Driver and front passenger airbag Supplemental Restraint

System (SRS) with 2-stage deployment and occupant sensor on the driver and passenger side.

Side-impact airbag Supplemental Restraint System (SRS) for front seats

Window curtain airbags Supplemental Restraint System (SRS) front and rear

Front and rear outboard seatbelt pretensioner and force limiter system

Keyless remote entry with adjustable audible warning, trunk release and panic alarm

Variable intermittent wipers and rain sensing wipers

Integrated fog lamps

First aid kit

3-point rear seat centre lap and shoulderbelt

Full-size spare tire

Key lockout protection

# Optional Equipment

LS 430 Premium Package includes: “Standard Features” plus Mark Levinson Audio, 240-watt system with 9 speakers and Radio Data System (RDS). Perforated leather seat surfaces. Laminated side glass, front seat heater and A/C, Laser cruise control, Auto Door Closer. (LS 430 Premium Package also available with DVD-based GPS Navigation System).

LS 430 Ultra Premium Package includes: “Standard Features” plus chrome alloy wheels, Mark Levinson Audio, 240-watt system with 9 speakers and Radio Data System (RDS). Air Suspension, laminated side glass, front seat heater and A/C, Laser Cruise, Auto Door Closer, sunshade on rear doors, rear A/C with climate control, rear power seats with massage setting, rear audio controls, DVD-based GPS Navigation System.

# Available Accessories

Block heater

# Warranty

Comprehensive coverage:

48 months/80,000 km (includes tires)

Powertrain: 72 months/110,000 km

Corrosion Perforation: 72 months/unlimited mileage

Roadside Assistance: 48 month, 24 hour roadside

assistance with free travel planning

Please see your Lexus dealer for details.

*TOYOTA CANADA INC. Every effort has been made to ensure the specifications and equipment shown are accurate based on information available at time of printing. In some cases, certain changes in standard equipment, options or product delays may occur, which would not be reflected in this brochure. Toyota Canada Inc. reserves the right to make these changes without notice or obligation. Your Lexus dealer is your best source for up-to-date information.

**Performance figure is shown for comparison only, and was obtained with prototype vehicles under test track conditions by professional drivers using special safety equipment and procedures. This should not be attempted on public streets or highways.

***FUEL CONSUMPTION RATING. These estimates are based on the Government of Canada’s approved criteria and testing methods, using 2002 model year vehicles. The actual fuel consumption of this vehicle may vary. Refer to the government of Canada publication – EnerGuide Fuel Consumption Guide.

Some features mentioned and/or shown in this brochure may only be available as part of an optional package.