# Lexus LX 470 Specifications & Features*

## Dimensions

| Body Type | 4-door, 8-passenger luxury sport utility with full-time 4WD. |
| --- | --- |
| Construction | Welded steel body on boxed-section ladder frame. |
| Overall Length | 4890 mm (192.5 in.) |
| Width | 1940 mm (76.4 in.) |
| Height (unloaded) | 1850 mm (72.8 in.) |
| Wheelbase | 2850 mm (112.2 in.) |
| Curb Weight | 2450 kg (5401 lbs.) |
| Gross Vehicle Weight | 3110 kg (6856 lbs.) |
| Tread (front/rear) | 1620/1615 mm (63.8/63.6 in.) |
| Ground Clearance | 220 mm (8.66 in.) at rear differential |
| Approach Angle | 30 degrees (“normal” height setting) 32 degrees (“high” height setting) |
| Departure Angle | 23 degrees (“normal” height setting) 27 degrees (“high” height setting) |
| Headroom (front/2nd row/3rd row) | 993/988/921 mm (39.1/38.9/36.3 in.) |
| Legroom (front/2nd row/3rd row) | 1074/870/694 mm (42.3/34.2/27.3 in.) |
| Shoulder Room (front/2nd row/3rd row) | 1558/1552/1553 mm (61.3/61.1/61.2 in.) |
| Hip Room (front/2nd row/3rd row) | 1488/1471/1432 mm (58.6/57.9/56.4 in.) |
| Cargo Capacity | Behind front seats (2nd row folded/3rd row removed) – 2.51 cu. m (88.6 cu. ft.) Behind 2nd row seats (3rd row seats removed) – 1.57 cu. m (55.4 cu. ft.) Behind 3rd row seats – .51 cu. m (18.0 cu. ft.) |
| Fuel tank capacity | 96.0 L (21.1 imp. gal.) |

## Engine

| Engine Type (2UZ-FE) | V8 |
| --- | --- |
| Displacement | 4.7 litres (4664cc) |
| Valvetrain | DOHC / 32 valves |
| Electronic Throttle Control System with intelligence (ETCS-i). |  |
| Bore x Stroke | 94 x 84 mm (3.70 x 3.31 in.) |
| Horsepower | 235 hp (175 kW) @ 4800 rpm |
| Torque | 320 lb. ft. (434 N.m) @ 3400 rpm |
| Compression Ratio | 9.6:1 |
| Engine Management | Computer control of Sequential multi-port fuel injection and Direct Ignition System (DIS) with Iridium-tipped spark plugs. |
| Fuel Requirement | Unleaded premium recommended. |

## Drivetrain

| Transmission | 5-speed Super Electronically-Controlled Transmission with intelligence (ECT-i). |
| --- | --- |
| Drive Wheels | Full-time 4-wheel. Centre differential provides controlled slip for differential action on high grip surfaces. |
| Final Drive Ratio | 4.30:1 |
| Active Traction Control (A-TRAC) |  |
| Vehicle Skid Control (VSC) |  |
| Suspension | Adaptive Variable Suspension (AVS) with adjustable ride height control. Front: Double wishbone with lower torsion bars. Rear: 4-link live axle with coil springs, stabilizer bar. |
| Steering | Engine speed-sensing variable gear ratio power-assisted rack-and-pinion, 3.8 turns lock-to-lock. |
| Brakes | 4-wheel, power assisted, ventilated discs (311 mm front/327 mm rear) with 4 piston front calipers. |
| Antilock Braking System (ABS) | 4-sensor, 4-channel. |
| Wheels/Tires | 18" aluminum alloy wheels, P275/60R18 M/S radials, full-size spare, wheel locks |
| Electronic Brake force Distribution (EBD) |  |
| Brake Assist |  |

## Performance Capabilities

| Fuel Consumption, L/100 km ** | 17.9 City, 12.9 Highway, 15.7 Combined |
| --- | --- |
| Turning Circle | 12.1 m (39.7 ft.) |
| Coefficient of Drag (cd.) | 0.40 |
| Towing Capacity | 2,268 kg (5,000 lbs.) |

## Interior/Exterior Features

Leather package includes: seat surfaces, door armrests & trim, centre console box cover, headrests, leather and wood shift knob and steering wheel

Mark Levinson audio system AM/FM ETR with cassette, 240 watt sound system with 9 speakers, power/ diversity antenna and steering wheel audio controls

In-dash 6-disc CD auto-changer

DVD-based GPS navigation system

Walnut trim

Dual stage driver and front passenger airbag

Supplemental Restraint System (SRS)

Side-impact airbag Supplemental Restraint System (SRS) for front seats

Front and middle seat side curtain airbag Supplemental Restraint System (SRS) with rollover sensors

Driver and front passenger seatbelt pretensioners/force limiters

3-point lap belts/shoulderbelts, front, 2nd row and outboard 3rd row, middle 3rd row lap belt

Adjustable front and outboard 2nd row shoulderbelt anchors

Front and rear passenger assist grips

Dual heated power remote-controlled folding and colour-keyed side mirrors with electrochromic automatic dimming & reverse auto tilt-down Electrochromic rear view mirror with integrated digital compass

Fully-integrated theft-deterrent system with encoded engine immobilizer

Power windows with “auto up/down”, anti-jam feature and retained accessory power

Power swing-out rear quarter windows

Power door locks with driver-door 2 turn unlock and “key-lockout” protection

Keyless remote entry system

Multi-adjustable power assisted and heated driver and front passenger seats with fold-down armrests

60/40 split and fold-down 2nd row seats with fold-down armrest and recline

50/50 split/foldable/removable 3rd row seat

Two-setting memory system: driver’s seat, steering wheel, exterior mirrors

Illuminated and lockable glove box

Overhead console with integrated garage door opener

Power tilt-and-slide moonroof with sunshade

Automatic climate control with dual air conditioning, “Soft-Touch” controls and dust, pollen and deodorizing air filters

Auxiliary rear heater

Illuminated entry system

Cruise Control

Tachometer, coolant temperature, oil pressure and voltmeter gauges

Electronic analog gauges

Digital clock

Outside temperature gauge

Driver and front passenger illuminated vanity mirrors

Privacy glass on rear doors, rear quarter windows and tailgate

Projector halogen headlamps with washer system

Auto-off headlamps and light control system

Projector fog lamps

Power tilt-and-telescopic steering wheel with automatic tilt-away

Heavy duty rear window defogger with timer

Rain sensing, variable intermittent wipers

Rear (Intermittent) wiper & washer

Illuminated running boards

Rear mudguards

Carpeted floor mats

Cargo area mat

First aid kit and tool kit

Remote fuel door release

High solar energy absorbing tinted glass

Roof rack

Two-piece lift up/pull-down tailgate

Spoiler

Towing receiver hitch

## Available Accessories

Ski/snowboard/bike carrier

Block heater

Cargo net

## Warranty

Comprehensive coverage: 48 months/80,000 km (includes tires)

Powertrain: 72 months/110,000 km

Corrosion Perforation: 72 months/unlimited mileage

Roadside Assistance: 48 month, 24 hour roadside assistance with free travel planning

Please see your Lexus dealer for details.

*TOYOTA CANADA INC. Every effort has been made to ensure the specifications and equipment shown are accurate based on information available at time of printing. In some cases, certain changes in standard equipment, options or product delays may occur, which would not be reflected in this brochure. Toyota Canada Inc. reserves the right to make these changes without notice or obligation. Your Lexus dealer is your best source for up-to-date information.

**FUEL CONSUMPTION RATING. These estimates are based on the Government of Canada’s approved criteria and testing methods. The actual fuel consumption of this vehicle may vary. Refer to the Government of Canada publication – EnerGuide Fuel Consumption Guide.

Some features mentioned may only be available as part of an optional package.